CSE 306 – COMPUTER NETWORKS
DETAILED LONG ANSWERS (FULL LENGTH NOTES)

Q1. Explain the OSI 7-Layer Model in detail.

Answer:
The OSI Model (Open Systems Interconnection) is a conceptual framework that standardizes how data travels from one device to another in a network. It divides communication into **7 layers**, each with specific functions.

1. **Physical Layer:** Deals with raw bit transmission through cables, voltages, frequencies, connectors. Converts data to electrical/optical signals.
2. **Data Link Layer:** Ensures reliable frame delivery between two directly connected nodes. Handles MAC addressing, framing, and error detection. Contains LLC and MAC sublayers.
3. **Network Layer:** Handles logical addressing (IP), routing, packet forwarding, fragmentation. Determines best path.
4. **Transport Layer:** Provides end‑to‑end communication using TCP/UDP. Ensures reliability, sequencing, flow control, error recovery.
5. **Session Layer:** Establishes, maintains, and terminates communication sessions. Manages checkpoints and synchronization.
6. **Presentation Layer:** Translates data between application and network. Encryption, compression, encoding/decoding happens here.
7. **Application Layer:** User interactable services such as HTTP, DNS, FTP, SMTP, DHCP.

The OSI model helps in troubleshooting, protocol design, and understanding how network layers interact.

Q2. Compare OSI and TCP/IP Models with diagrams.

Answer:
The **OSI Model** is theoretical and has 7 layers; the **TCP/IP Model** is practical and has 4 layers. OSI is used for understanding and teaching, while TCP/IP is used for real-world internet communication.

**OSI Layers:** Physical, Data Link, Network, Transport, Session, Presentation, Application.
**TCP/IP Layers:** Network Access, Internet, Transport, Application.

**Major Differences:**
- OSI separates session and presentation; TCP/IP merges them inside Application.
- OSI is protocol independent; TCP/IP uses specific protocols (IP, TCP, UDP).
- TCP/IP is more flexible and widely used.

**Similarities:** Both are layered, support communication, and provide interoperability.

OSI focuses on function; TCP/IP focuses on protocols.

Q3. Explain different Network Topologies with diagrams and advantages.

Answer:
Network topology is the physical or logical arrangement of devices.

1. **Bus Topology:** A single central cable connects all devices. Cheap, easy, but collisions occur and cable failure breaks network.
2. **Star Topology:** All devices connect to a hub/switch. Easy to troubleshoot, scalable, but central device failure stops network.
3. **Ring Topology:** Devices form a circular path. Data flows in one direction. Predictable but entire ring breaks if one node fails.
4. **Mesh Topology:** Each device connects to every other device. Highly reliable, no single point of failure, but expensive and complex.
5. **Tree Topology:** Combination of multiple star networks arranged hierarchically. Good for large networks but complex cabling.
6. **Hybrid Topology:** Combines two or more topologies for flexibility. Very powerful but costly.

Topologies affect performance, reliability, cost, and manageability.

Q4. Explain Analog & Digital Signals and signal impairments.

Answer:
**Analog Signals** are continuous waves, vary in amplitude and frequency. Used in audio, broadcast, telephone.
**Digital Signals** are discrete binary values (0 and 1), easy to process and noise-resistant.

**Signal Impairments:**
1. **Attenuation:** Loss of signal strength over distance. Fixed using amplifiers.
2. **Distortion:** Shape changes due to different propagation speeds.
3. **Noise:** Unwanted signals causing errors (thermal, impulse, crosstalk).

Impairments cause data corruption and require techniques like encoding, shielding, repeaters.

Q5. What is Multiplexing? Explain FDM, TDM, WDM.

Answer:
Multiplexing allows multiple signals to share a single communication medium to improve efficiency.

**Types:**
1. **FDM:** Different frequency bands assigned to each signal. Used in radio/TV.
2. **TDM:** Time slots assigned to each signal. Used in telephone networks.
3. **WDM:** Different wavelengths used in optical fiber. High-speed backbone networks use it.

Multiplexing reduces cost and increases efficiency.

Q6. Explain Error Detection & Correction Techniques.

Answer:
Communication systems face errors, so detection and correction methods are required.

**Detection Techniques:**
- **Parity Bit:** Simple even/odd parity check.
- **Checksum:** Adds all segments; receiver verifies sum.
- **CRC:** Uses polynomial division; very accurate.

**Correction Technique:**
- **Hamming Code:** Detects and corrects single-bit errors using redundant bits.

Reliable communication needs strong error control.

Q7. Explain ALOHA, Slotted ALOHA, CSMA, CSMA/CD.

Answer:
**ALOHA:** Nodes transmit whenever they want. High collisions, low efficiency.

**Slotted ALOHA:** Time is divided into slots. Nodes transmit only at slot boundaries. Efficiency nearly doubled.

**CSMA:** Carrier Sensing. Nodes first check if channel is idle before sending.

**CSMA/CD:** Used in Ethernet. Detects collisions, sends jam signal, waits random backoff time.

These protocols prevent multiple nodes from corrupting data.

Q8. Explain IPv4 Addressing, Subnetting, and CIDR with examples.

Answer:
IPv4 is a 32‑bit address format divided into network and host portions.

**Subnetting:** Dividing network into smaller networks. Example:
192.168.1.0/24 → 256 addresses.

**CIDR:** Classless addressing removes class A/B/C limits. Example:
10.0.0.0/12 → large block but flexible.

Subnetting improves security, reduces broadcast traffic, and improves routing.

Q9. Describe Distance Vector & Link-State Routing Algorithms.

Answer:
**Distance Vector:** Uses Bellman-Ford. Routers exchange tables with neighbors. Slow convergence, vulnerable to count-to-infinity.

**Link-State:** Uses Dijkstra. Routers build full topology of network using LSA packets. Fast, reliable, used in OSPF.

LS is preferred for modern large-scale networks; DV is simpler for small networks.

Q10. Explain TCP: header, 3-way handshake, flow control & congestion control.

Answer:
TCP provides reliable connection-oriented service.

**TCP Header Fields:** Source port, destination port, sequence number, acknowledgment number, flags, window size, checksum.

**3-Way Handshake:**
1. Client → SYN
2. Server → SYN-ACK
3. Client → ACK
Connection established.

**Flow Control:** Sliding window controls data rate.

**Congestion Control:** Slow start, congestion avoidance, fast retransmission, fast recovery.

TCP ensures ordered, loss-free communication.